## The Attack

Malicious actors can take advantage of the TCP protocol by flooding a server with SYN packet requests for the first part of the handshake. However, if the number of SYN requests is greater than the server resources available to handle the requests, then the server will become overwhelmed and unable to respond to the requests. This is a network level denial of service (DoS) attack, called a SYN flood attack, that targets network bandwidth to slow traffic. A SYN flood attack simulates a TCP connection and floods the server with SYN packets. A DoS direct attack originates from a single source. A distributed denial of service (DDoS) attack comes from multiple sources, often in different locations, making it more difficult to identify the attacker or attackers.

There are two tabs at the bottom of the log file. One is labeled “Color coded TCP log.” If you click on that tab, you will find the server interactions with the attacker’s IP address (203.0.113.0) marked with red highlighting (and the word “red” in column A).

| Color as text | No. | Time | Source (x = redacted) | Destination (x = redacted) | Protocol | Info |
| --- | --- | --- | --- | --- | --- | --- |
| red | 52 | 3.390692 | 203.0.113.0 | 192.0.2.1 | TCP | 54770->443 [SYN] Seq=0 Win=5792 Len=0... |
| red | 53 | 3.441926 | 192.0.2.1 | 203.0.113.0 | TCP | 443->54770 [SYN, ACK] Seq=0 Win-5792 Len=120... |
| red | 54 | 3.493160 | 203.0.113.0 | 192.0.2.1 | TCP | 54770->443 [ACK Seq=1 Win=5792 Len=0... |
| green | 55 | 3.544394 | 198.51.100.14 | 192.0.2.1 | TCP | 14785->443 [SYN] Seq=0 Win-5792 Len=120... |
| green | 56 | 3.599628 | 192.0.2.1 | 198.51.100.14 | TCP | 443->14785 [SYN, ACK] Seq=0 Win-5792 Len=120... |
| red | 57 | 3.664863 | 203.0.113.0 | 192.0.2.1 | TCP | 54770->443 [SYN] Seq=0 Win=5792 Len=0... |
| green | 58 | 3.730097 | 198.51.100.14 | 192.0.2.1 | TCP | 14785->443 [ACK] Seq=1 Win-5792 Len=120... |
| red | 59 | 3.795332 | 203.0.113.0 | 192.0.2.1 | TCP | 54770->443 [SYN] Seq=0 Win-5792 Len=120... |
| green | 60 | 3.860567 | 198.51.100.14 | 192.0.2.1 | HTTP | GET /sales.html HTTP/1.1 |
| red | 61 | 3.939499 | 203.0.113.0 | 192.0.2.1 | TCP | 54770->443 [SYN] Seq=0 Win-5792 Len=120... |
| green | 62 | 4.018431 | 192.0.2.1 | 198.51.100.14 | HTTP | HTTP/1.1 200 OK (text/html) |

Initially, the attacker’s SYN request is answered normally by the web server (log items 52-54). However, the attacker keeps sending more SYN requests, which is abnormal. At this point, the web server is still able to respond to normal visitor traffic, which is highlighted and labeled as green. An employee visitor with the IP address of 198.51.100.14 successfully completes a SYN/ACK connection handshake with the webserver (log item nos. 55, 56, 58). Then, the employee’s browser requests the sales.html webpage with the GET command and the web server responds (log item no. 60 and 62).

| Color as text | No. | Time | Source | Destination | Protocol | Info |
| --- | --- | --- | --- | --- | --- | --- |
| green | 63 | 4.097363 | 198.51.100.5 | 192.0.2.1 | TCP | 33638->443 [SYN] Seq=0 Win-5792 Len=120... |
| red | 64 | 4.176295 | 192.0.2.1 | 203.0.113.0 | TCP | 443->54770 [SYN, ACK] Seq=0 Win-5792 Len=120... |
| green | 65 | 4.255227 | 192.0.2.1 | 198.51.100.5 | TCP | 443->33638 [SYN, ACK] Seq=0 Win-5792 Len=120... |
| red | 66 | 4.256159 | 203.0.113.0 | 192.0.2.1 | TCP | 54770->443 [SYN] Seq=0 Win=5792 Len=0... |
| green | 67 | 5.235091 | 198.51.100.5 | 192.0.2.1 | TCP | 33638->443 [ACK] Seq=1 Win-5792 Len=120... |
| red | 68 | 5.236023 | 203.0.113.0 | 192.0.2.1 | TCP | 54770->443 [SYN] Seq=0 Win=5792 Len=0... |
| green | 69 | 5.236955 | 198.51.100.16 | 192.0.2.1 | TCP | 32641->443 [SYN] Seq=0 Win-5792 Len=120... |
| red | 70 | 5.237887 | 203.0.113.0 | 192.0.2.1 | TCP | 54770->443 [SYN] Seq=0 Win=5792 Len=0... |
| green | 71 | 6.228728 | 198.51.100.5 | 192.0.2.1 | HTTP | GET /sales.html HTTP/1.1 |
| red | 72 | 6.229638 | 203.0.113.0 | 192.0.2.1 | TCP | 54770->443 [SYN] Seq=0 Win=5792 Len=0... |
| yellow | 73 | 6.230548 | 192.0.2.1 | 198.51.100.16 | TCP | 443->32641 [RST, ACK] Seq=0 Win-5792 Len=120... |
| red | 74 | 6.330539 | 203.0.113.0 | 192.0.2.1 | TCP | 54770->443 [SYN] Seq=0 Win=5792 Len=0... |
| green | 75 | 6.330885 | 198.51.100.7 | 192.0.2.1 | TCP | 42584->443 [SYN] Seq=0 Win=5792 Len=0... |
| red | 76 | 6.331231 | 203.0.113.0 | 192.0.2.1 | TCP | 54770->443 [SYN] Seq=0 Win=5792 Len=0... |
| yellow | 77 | 7.330577 | 192.0.2.1 | 198.51.100.5 | TCP | HTTP/1.1 504 Gateway Time-out (text/html) |
| red | 78 | 7.331323 | 203.0.113.0 | 192.0.2.1 | TCP | 54770->443 [SYN] Seq=0 Win=5792 Len=0... |
| green | 79 | 7.340768 | 198.51.100.22 | 192.0.2.1 | TCP | 6345->443 [SYN] Seq=0 Win=5792 Len=0... |
| yellow | 80 | 7.340773 | 192.0.2.1 | 198.51.100.7 | TCP | 443->42584 [RST, ACK] Seq=1 Win-5792 Len=120... |
| red | 81 | 7.340778 | 203.0.113.0 | 192.0.2.1 | TCP | 54770->443 [SYN] Seq=0 Win=5792 Len=0... |
| red | 82 | 7.340783 | 203.0.113.0 | 192.0.2.1 | TCP | 54770->443 [SYN] Seq=0 Win=5792 Len=0... |
| red | 83 | 7.439658 | 192.0.2.1 | 203.0.113.0 | TCP | 443->54770 [RST, ACK] Seq=1 Win=5792 Len=0... |

In the next 20 rows, the log begins to reflect the struggle the web server is having to keep up with the abnormal number of SYN requests coming in at a rapid pace. The attacker is sending several SYN requests every second. The rows highlighted and labeled yellow are failed communications between legitimate employee website visitors and the web server.

The two types of errors in the logs include:

- An HTTP/1.1 504 Gateway Time-out (text/html) error message. This message is generated by a gateway server that was waiting for a response from the web server. If the web server takes too long to respond, the gateway server will send a timeout error message to the requesting browser.

- An [RST, ACK] packet, which would be sent to the requesting visitor if the [SYN, ACK] packet is not received by the web server. RST stands for reset, acknowledge. The visitor will receive a timeout error message in their browser and the connection attempt is dropped. The visitor can refresh their browser to attempt to send a new SYN request.

| Color as text | No. | Time | Source (x = redacted) | Destination (x = redacted) | Protocol | Info |
| --- | --- | --- | --- | --- | --- | --- |
| red | 119 | 19.198705 | 203.0.113.0 | 192.0.2.1 | TCP | 54770->443 [SYN] Seq=0 Win=5792 Len=0... |
| red | 120 | 19.521718 | 203.0.113.0 | 192.0.2.1 | TCP | 54770->443 [SYN] Seq=0 Win=5792 Len=0... |
| yellow | 121 | 19.844731 | 192.0.2.1 | 198.51.100.9 | TCP | 443->4631 [RST, ACK] Seq=1 Win=5792 Len=0... |
| red | 122 | 20.167744 | 203.0.113.0 | 192.0.2.1 | TCP | 54770->443 [SYN] Seq=0 Win=5792 Len=0... |
| red | 123 | 20.490757 | 203.0.113.0 | 192.0.2.1 | TCP | 54770->443 [SYN] Seq=0 Win=5792 Len=0... |
| red | 124 | 20.81377 | 192.0.2.1 | 203.0.113.0 | TCP | 443->54770 [RST, ACK] Seq=1 Win=5792 Len=0... |
| red | 125 | 21.136783 | 203.0.113.0 | 192.0.2.1 | TCP | 54770->443 [SYN] Seq=0 Win=5792 Len=0... |
| red | 126 | 21.459796 | 203.0.113.0 | 192.0.2.1 | TCP | 54770->443 [SYN] Seq=0 Win=5792 Len=0... |
| red | 127 | 21.782809 | 203.0.113.0 | 192.0.2.1 | TCP | 54770->443 [SYN] Seq=0 Win=5792 Len=0... |
| red | 128 | 22.105822 | 203.0.113.0 | 192.0.2.1 | TCP | 54770->443 [SYN] Seq=0 Win=5792 Len=0... |
| red | 129 | 22.428835 | 203.0.113.0 | 192.0.2.1 | TCP | 54770->443 [SYN] Seq=0 Win=5792 Len=0... |
| red | 130 | 22.751848 | 203.0.113.0 | 192.0.2.1 | TCP | 54770->443 [SYN] Seq=0 Win=5792 Len=0... |
| red | 131 | 23.074861 | 203.0.113.0 | 192.0.2.1 | TCP | 54770->443 [SYN] Seq=0 Win=5792 Len=0... |
| red | 132 | 23.397874 | 203.0.113.0 | 192.0.2.1 | TCP | 54770->443 [SYN] Seq=0 Win=5792 Len=0... |
| red | 133 | 23.720887 | 203.0.113.0 | 192.0.2.1 | TCP | 54770->443 [SYN] Seq=0 Win=5792 Len=0... |
| red | 134 | 24.0439 | 203.0.113.0 | 192.0.2.1 | TCP | 54770->443 [SYN] Seq=0 Win=5792 Len=0... |
| red | 135 | 24.366913 | 203.0.113.0 | 192.0.2.1 | TCP | 54770->443 [SYN] Seq=0 Win=5792 Len=0... |
| red | 136 | 24.689926 | 203.0.113.0 | 192.0.2.1 | TCP | 54770->443 [SYN] Seq=0 Win=5792 Len=0... |
| red | 137 | 25.012939 | 203.0.113.0 | 192.0.2.1 | TCP | 54770->443 [SYN] Seq=0 Win=5792 Len=0... |
| red | 138 | 25.335952 | 203.0.113.0 | 192.0.2.1 | TCP | 54770->443 [SYN] Seq=0 Win=5792 Len=0... |
| red | 139 | 25.658965 | 203.0.113.0 | 192.0.2.1 | TCP | 54770->443 [SYN] Seq=0 Win=5792 Len=0... |
| red | 140 | 25.981978 | 203.0.113.0 | 192.0.2.1 | TCP | 54770->443 [SYN] Seq=0 Win=5792 Len=0... |
| red | 141 | 26.304991 | 203.0.113.0 | 192.0.2.1 | TCP | 54770->443 [SYN] Seq=0 Win=5792 Len=0... |
| red | 142 | 26.628004 | 203.0.113.0 | 192.0.2.1 | TCP | 54770->443 [SYN] Seq=0 Win=5792 Len=0... |
| red | 143 | 26.951017 | 203.0.113.0 | 192.0.2.1 | TCP | 54770->443 [SYN] Seq=0 Win=5792 Len=0... |
| red | 144 | 27.27403 | 203.0.113.0 | 192.0.2.1 | TCP | 54770->443 [SYN] Seq=0 Win=5792 Len=0... |
| red | 145 | 27.597043 | 203.0.113.0 | 192.0.2.1 | TCP | 54770->443 [SYN] Seq=0 Win=5792 Len=0... |
| red | 146 | 27.920056 | 203.0.113.0 | 192.0.2.1 | TCP | 54770->443 [SYN] Seq=0 Win=5792 Len=0... |
| red | 147 | 28.243069 | 203.0.113.0 | 192.0.2.1 | TCP | 54770->443 [SYN] Seq=0 Win=5792 Len=0... |
| red | 148 | 28.566082 | 203.0.113.0 | 192.0.2.1 | TCP | 54770->443 [SYN] Seq=0 Win=5792 Len=0... |
| red | 149 | 28.889095 | 203.0.113.0 | 192.0.2.1 | TCP | 54770->443 [SYN] Seq=0 Win=5792 Len=0... |
| red | 150 | 29.212108 | 203.0.113.0 | 192.0.2.1 | TCP | 54770->443 [SYN] Seq=0 Win=5792 Len=0... |
| red | 151 | 29.535121 | 203.0.113.0 | 192.0.2.1 | TCP | 54770->443 [SYN] Seq=0 Win=5792 Len=0... |
| red | 152 | 29.858134 | 203.0.113.0 | 192.0.2.1 | TCP | 54770->443 [SYN] Seq=0 Win=5792 Len=0... |

As you scroll through the rest of the log, you will notice the web server stops responding to  legitimate employee visitor traffic. The visitors receive more error messages indicating that they cannot establish or maintain a connection to the web server. From log item number 125 on, the web server stops responding. The only items logged at that point are from the attack. As there is only one IP address attacking the web server, you can assume this is a direct DoS SYN flood attack.